DATA COLLECTION PLAN

Topology-Aware Federated Edge Learning for DER Intrusion Detection

Instructions for Data Collection Colleague

# 1. Overview

This document tells you exactly what data to collect, how to collect it, and what the output files should look like. Read the entire document before running any simulation.

You need to run 3 simulations total using PandaPower on the IEEE 33-Bus system:

| Run | DER Active? | Attacks? | Duration | Purpose |
| --- | --- | --- | --- | --- |
| Run 1A | NO | No | 4 weeks | Benchmarking baseline (no DER) |
| Run 1B | YES | No | 4 weeks | Model training data (with DER) |
| Run 2 | YES | Replay injected after | 2 weeks | Testing data for all models |

NOTE: Run 1A and Run 1B must use the same time period and same load profiles. The ONLY difference is DER is switched off in Run 1A.

# 2. One-Time Admittance Extraction

Admittance is a fixed property of the power lines. It never changes between simulation runs. Run this script ONCE, save the output file, and never run it again.

Code to run:

import pandapower as pp

import pandas as pd

import numpy as np

net = pp.networks.case33bw()

net.line['z'] = (net.line['r_ohm_per_km'] +

1j * net.line['x_ohm_per_km']) * net.line['length_km']

net.line['admittance'] = 1 / net.line['z'].abs()

net.line[['from_bus', 'to_bus', 'admittance']].to_csv('zone_admittance.csv', index=False)

Output file — zone_admittance.csv:

| from_bus | to_bus | admittance |
| --- | --- | --- |
| 0 | 1 | 12.453 |
| 1 | 2 | 8.231 |
| 2 | 3 | 9.847 |
| ... | ... | ... |

IMPORTANT: Save this file. It is needed by the training pipeline later.

# 3. Zone Partitioning

The 33 buses are split into 4 zones based on electrical proximity. Use these exact zone definitions for all simulation runs:

| Zone | Bus Numbers | DER Present | DER Bus |
| --- | --- | --- | --- |
| Zone 1 | 0, 1, 2, 3, 4, 5, 6, 7, 8 | No | - |
| Zone 2 | 9, 10, 11, 12, 13, 14, 15, 16, 17 | Yes (DER_1) | Bus 17 |
| Zone 3 | 18, 19, 20, 21, 22, 23, 24 | Yes (DER_2) | Bus 24 |
| Zone 4 | 25, 26, 27, 28, 29, 30, 31, 32 | Yes (DER_3) | Bus 32 |

# 4. Simulation Settings

## 4.1 General Settings (Apply to ALL Runs)

- Time resolution: 5-minute intervals

- Features to record per bus: V_PU, P_MW, Q_MVAR, THETA_DEG

- Record ALL 33 buses at every timestep

- Output format: long format CSV (33 rows per timestamp, one per bus)

## 4.2 Run 1A Settings (No DER Baseline)

- Set der_output_mw = 0 for ALL DER buses (17, 24, 32)

- Set der_output_mvar = 0 for ALL DER buses

- Keep has_der flag as-is in the data — just zero the output

- Duration: 4 weeks (July 1 to July 28, 2024 recommended)

## 4.3 Run 1B Settings (With DER — Training)

- DER fully active — solar generation follows normal profile

- Same time period as Run 1A (July 1 to July 28, 2024)

- Same load profiles as Run 1A

- Duration: 4 weeks

## 4.4 Run 2 Settings (With DER — Testing Base)

- DER fully active

- Different time period from training (August 1 to August 14, 2024)

- Simulate normally — DO NOT inject attacks during simulation

- Attacks are injected AFTER simulation as a post-processing step (see Section 6)

- Duration: 2 weeks

# 5. Raw Simulation Output Format

Every simulation run outputs a single long-format CSV. Each timestamp produces 33 rows, one per bus. This is the raw format before any reshaping.

Raw CSV columns to keep:

| Column | Description | Keep? |
| --- | --- | --- |
| timestamp | Datetime of measurement | YES |
| bus_id | Bus number (0-32) | YES |
| V_PU | Voltage magnitude in per unit | YES |
| P_MW | Active power in megawatts | YES |
| Q_MVAR | Reactive power in megavars | YES |
| THETA_DEG | Voltage phase angle in degrees | YES |
| load_multiplier | Simulation input parameter | DROP |
| solar_multiplier | Simulation input parameter | DROP |
| weather | Simulation input parameter | DROP |
| has_der | Simulation metadata | DROP |
| der_name | Simulation metadata | DROP |
| der_capacity_mw | Simulation metadata | DROP |
| der_output_mw | Simulation metadata | DROP |
| der_output_mvar | Simulation metadata | DROP |
| hour, day_of_week, etc. | Derived columns | DROP |

NOTE: Keep only the 6 columns marked YES. Drop everything else before reshaping.

# 6. Reshaping: Long Format to Wide Format

After each simulation run, reshape the raw CSV from long format (33 rows per timestamp) to wide format (1 row per timestamp, all buses side by side). Do this separately for each zone.

Reshaping code:

import pandas as pd

df = pd.read_csv('simulation_output.csv')

# Keep only needed columns

df = df[['timestamp', 'bus_id', 'V_PU', 'P_MW', 'Q_MVAR', 'THETA_DEG']]

zones = {

'zone1': list(range(0, 9)),

'zone2': list(range(9, 18)),

'zone3': list(range(18, 25)),

'zone4': list(range(25, 33))

}

features = ['V_PU', 'P_MW', 'Q_MVAR', 'THETA_DEG']

for zone_name, buses in zones.items():

zone_df = df[df['bus_id'].isin(buses)]

wide = zone_df.pivot(index='timestamp', columns='bus_id', values=features)

wide.columns = [f'{feat}_bus{bus}' for feat, bus in wide.columns]

wide = wide.reset_index()

wide.to_csv(f'{zone_name}_output.csv', index=False)

Wide format looks like this (Zone 1 example):

| timestamp | V_bus0 | P_bus0 | Q_bus0 | THETA_bus0 | V_bus1 | ... | THETA_bus8 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1/7/2024 0:00 | 1.000 | -2.267 | -1.407 | 0.000 | 0.998 | ... | -0.033 |
| 1/7/2024 0:05 | 1.000 | -2.251 | -1.397 | 0.000 | 0.998 | ... | -0.033 |
| 1/7/2024 0:10 | 1.000 | -2.243 | -1.391 | 0.000 | 0.998 | ... | -0.034 |
| ... | ... | ... | ... | ... | ... | ... | ... |

NOTE: Zone 2 will have 9 buses x 4 features = 36 feature columns. Zone 3 has 7 buses x 4 = 28 columns. Zone 4 has 8 buses x 4 = 32 columns.

# 7. Training Files (From Run 1B)

After reshaping Run 1B output, you get 4 training files. These are used to train the LSTM Autoencoder on NORMAL behavior only.

- No attack_label column — these files contain normal data only

- Do not shuffle rows — keep original time order

- 4 weeks of data = approximately 8,064 rows per zone file

| File | Buses Included | Approx Rows |
| --- | --- | --- |
| zone1_train.csv | Buses 0-8 | 8,064 |
| zone2_train.csv | Buses 9-17 (DER_1 at bus 17) | 8,064 |
| zone3_train.csv | Buses 18-24 (DER_2 at bus 24) | 8,064 |
| zone4_train.csv | Buses 25-32 (DER_3 at bus 32) | 8,064 |

NOTE: Run 1A (no DER) files are reshaped the same way and saved as zone1_train_noder.csv etc. These are for benchmarking plots only, not model training.

# 8. Replay Attack Injection (Test Data)

After reshaping Run 2 output, inject replay attacks as a post-processing step. Do NOT modify the simulation — attacks are added to the CSV files after the fact.

## 8.1 How Replay Attack Works

A replay attack copies normal measurements from a low-demand period (midnight) and pastes them over a high-demand period (noon) in ONE zone only. The neighboring zones remain untouched.

IMPORTANT: Only ONE zone is attacked per attack event. Neighboring zones stay completely normal. This contrast is exactly what the detection system exploits.

## 8.2 Attack Injection Code

import pandas as pd

import numpy as np

def inject_replay(df, source_start, source_end, target_start, target_end):

df = df.copy()

df['attack_label'] = 0  # default: normal

feature_cols = [c for c in df.columns if c != 'timestamp']

# Copy low-demand window values over high-demand window

source = df[feature_cols].iloc[source_start:source_end].values

df.iloc[target_start:target_end,

df.columns.get_indexer(feature_cols)] = source

df.loc[target_start:target_end-1, 'attack_label'] = 1

return df

# Example: attack zone 2 at noon on day 3

# Midnight = rows 0-24 (12 intervals/hour x 2 hours)

# Noon day 3 = rows 864-888

zone2 = pd.read_csv('zone2_test_raw.csv')

zone2_attacked = inject_replay(zone2, 0, 24, 864, 888)

zone2_attacked.to_csv('zone2_test.csv', index=False)

# Neighboring zones: just add label column = 0

for zone in ['zone1', 'zone3', 'zone4']:

df = pd.read_csv(f'{zone}_test_raw.csv')

df['attack_label'] = 0

df.to_csv(f'{zone}_test.csv', index=False)

## 8.3 Attack Schedule

Inject 6 replay events spread across the 2-week test period. Vary the targeted zone and time of day:

| Event | Targeted Zone | Source Window | Target Window (approx) |
| --- | --- | --- | --- |
| Attack 1 | Zone 2 | Midnight Day 1 | Noon Day 3 |
| Attack 2 | Zone 4 | Midnight Day 2 | Noon Day 5 |
| Attack 3 | Zone 1 | Midnight Day 1 | Evening Day 7 |
| Attack 4 | Zone 3 | Midnight Day 3 | Noon Day 9 |
| Attack 5 | Zone 2 | Midnight Day 2 | Afternoon Day 11 |
| Attack 6 | Zone 4 | Midnight Day 1 | Noon Day 13 |

NOTE: Each attack event is 2 hours long (24 rows at 5-min intervals). This gives roughly 144 attack rows out of ~4,032 total test rows = about 3.5% attack density, which is realistic.

# 9. Test File Format

Test files look like training files but with an added attack_label column at the end. Label values: 0 = normal, 1 = replay attack.

Attacked zone test file (e.g. zone2_test.csv) during attack window:

| timestamp | V_bus9 | P_bus9 | ... | THETA_bus17 | attack_label |
| --- | --- | --- | --- | --- | --- |
| 1/8/2024 11:55 | 0.963 | 0.041 | ... | -0.275 | 0 (normal) |
| 1/8/2024 12:00 | 0.958 | 0.035 | ... | -0.271 | 1 (REPLAYED) |
| 1/8/2024 12:05 | 0.958 | 0.035 | ... | -0.271 | 1 (REPLAYED) |
| 1/8/2024 14:05 | 0.964 | 0.042 | ... | -0.276 | 0 (normal) |

Neighboring zone (e.g. zone1_test.csv) at the SAME time:

| timestamp | V_bus0 | P_bus0 | ... | THETA_bus8 | attack_label |
| --- | --- | --- | --- | --- | --- |
| 1/8/2024 12:00 | 1.000 | -2.891 | ... | -0.089 | 0 (normal, untouched) |
| 1/8/2024 12:05 | 1.000 | -2.887 | ... | -0.091 | 0 (normal, untouched) |

IMPORTANT: The contrast between zone2 showing label=1 while zone1 shows label=0 at the same timestamp is the key signal your detection system uses.

# 10. Final File Checklist

When you are done, you should have exactly these files:

| File | From Run | Has attack_label? | Purpose |
| --- | --- | --- | --- |
| zone_admittance.csv | One-time extraction | No | FL aggregation weights |
| zone1_train_noder.csv | Run 1A | No | Benchmarking plots only |
| zone2_train_noder.csv | Run 1A | No | Benchmarking plots only |
| zone3_train_noder.csv | Run 1A | No | Benchmarking plots only |
| zone4_train_noder.csv | Run 1A | No | Benchmarking plots only |
| zone1_train.csv | Run 1B | No | Train all models |
| zone2_train.csv | Run 1B | No | Train all models |
| zone3_train.csv | Run 1B | No | Train all models |
| zone4_train.csv | Run 1B | No | Train all models |
| zone1_test.csv | Run 2 + injection | YES | Test all models |
| zone2_test.csv | Run 2 + injection | YES | Test all models |
| zone3_test.csv | Run 2 + injection | YES | Test all models |
| zone4_test.csv | Run 2 + injection | YES | Test all models |

IMPORTANT: The SAME 4 test files are used to evaluate all three models: local (no FL), FedAvg, and topology-aware FedAvg. Never create separate test files per model.

# 11. Key Rules Summary

- Never shuffle rows in any file. Always preserve time order.

- Training files have NO attack_label column.

- Test files ALWAYS have an attack_label column (0 = normal, 1 = replay).

- Run 1A and Run 1B must use the same time period and same load profiles. Only DER output changes.

- Attacks are injected only into test files as post-processing. Never inject during simulation.

- Only one zone is attacked per event. Neighboring zones stay label=0.

- Admittance file is extracted once from PandaPower and never regenerated.

- Drop all simulation input columns (weather, multipliers, der metadata). Keep only: timestamp, bus_id, V_PU, P_MW, Q_MVAR, THETA_DEG.

If anything is unclear, ask before running — do not guess.